# G.U.M.B.O. — Great Utility for Music Box Optimization

## User Manual

## 1. What Is G.U.M.B.O.?

G.U.M.B.O. is a parametric OpenSCAD framework for designing 3D‑printable loudspeaker enclosures. It combines box sizing, driver layout, baffle geometry, porting, back‑panel hardware, and fit‑verification into a single coherent system.

G.U.M.B.O. is designed to: - Minimize trial‑and‑error printing - Keep all geometry driven by parameters - Let you see the entire speaker before committing to a print - Provide verification tools (fit test puck) so failures happen on-screen, not on the printer

If you can edit numbers and press F5, you can use G.U.M.B.O.

## 2. File Structure Overview

G.U.M.B.O. is modular by design.

- speakerGUMBO_main.scad
Orchestrates geometry, routing, and exports

- parameters.scad
All user‑editable design parameters (this is where you spend 90% of your time)

- drivers.scad
Driver cutouts, face shapes, screw patterns, and fit‑test puck

- baffle.scad
Baffle geometry, recesses, trim, and locators

- back_panel.scad
Terminal and port mounting surface

- utils.scad
Generic math and helper utilities (no project‑specific geometry)

You should never need to edit anything except parameters.scad during normal use.

## 3. Basic Workflow (Read This First)

A typical G.U.M.B.O. workflow looks like this:

- Set enclosure intent (sealed or ported)

- Define drivers (sizes, shapes, screws)

- Adjust box proportions

- Adjust driver placement

- Tune baffle geometry

- Verify fit using the fit‑test puck

- Export STL files

G.U.M.B.O. is fast enough that iteration is encouraged.

## 4. Global Display & Debug Controls

Located at the top of parameters.scad.

### $fn

Controls curve resolution. - 32–64: preview - 128+: final print

### showSpecs

Prints calculated dimensions, volumes, and tuning data to the console.

### showDebug

Shows driver center markers.

### showDriverFaces

Overlays driver face outlines on the baffle for visual confirmation.

These options never affect exported geometry.

## 5. Enclosure Type & Alignment

### sealed

- true → sealed box

- false → ported box

### Thiele‑Small Parameters (woofer)

- Vas – Equivalent compliance volume (liters)

- Qts – Total Q of the driver

- Qtc – Desired system Q (sealed only)

For ported designs, G.U.M.B.O. estimates volume using:

Vb ≈ Vas × ventedVasFactor

A safety clamp prevents invalid sealed alignments.

## 6. Box Geometry

### Wall Thickness

boxThickness controls structural rigidity and print time.

### Proportions

The box is sized by ratios, not fixed dimensions: - boxHeightRatio - boxWidthRatio - boxDepthRatio

This keeps volume consistent while allowing shape exploration.

### Edge Rounding

- round_horizontal_edges

- edge_round_radius

These affect both the box and front trim automatically.

## 7. Driver Definitions

Drivers are defined once and reused everywhere.

### driverParams

Each entry:

[cutout_diameter, screw_spacing, screw_diameter, enabled]

Driver index order is fixed: - 0 = Tweeter - 1 = Midrange - 2 = Woofer

### speakerFaceDiameters

Defines flange size for recesses and trim.

### driverScrewCount

Number of mounting screws per driver.

Disabling a driver removes it entirely from geometry.

## 8. Driver Face Shapes

G.U.M.B.O. supports eight face shapes:

| Code | Shape |
| --- | --- |
| 0 | Round |
| 1 | Square |
| 2 | Rounded square |
| 3 | Rectangle |
| 4 | Clipped circle (4 flats) |
| 5 | Top/bottom clipped |
| 6 | Squircle |
| 7 | Adjustable superellipse |

Shape‑specific parameters are grouped and only used when needed.

Shape 7 allows precise matching of modern PC/GRS‑style frames.

## 9. Driver Placement

Drivers are automatically spaced vertically, with optional manual offsets.

### Manual Offsets

- *_Z_offset – vertical adjustment

- *_X_offset – horizontal adjustment

### Safety

driverEdgeMin enforces minimum clearance to inner walls. Offsets are clamped automatically.

## 10. Baffle Geometry

The baffle is a separate part that mounts to the front of the box.

### Thickness & Shape

- baffleThickness

- baffleCornerR

- baffleEdgeStyle (flat / chamfer / roundover)

- baffleEdgeSize

### Mounting Style

- Flush mount (recess + trim)

- Surface mount

Each driver can be configured independently.

## 11. Baffle Locators

Glue‑on baffles use mechanical alignment features:

- Slots (one side)

- Keys (opposite side)

Parameters control size, depth, and clearance. These ensure repeatable alignment during assembly.

## 12. Ports

### Legacy Single Port

- Round, rear‑mounted

- Optional printed tube

- Optional flares

### Multi‑Port Mode

Supports 1–3 identical ports: - Auto‑centered - Even spacing - Shared tuning calculation

Helmholtz frequency is estimated automatically.

## 13. Terminals & Back Panel

Back panel is a removable, surface‑mounted plate.

Supports multiple terminal styles: - Wire holes - Cups - Binding posts - NL2 - Spring clips

Back panel thickness matches box walls for stiffness.

## 14. Fit Test Puck (Highly Recommended)

G.U.M.B.O. includes a flush‑fit verification puck.

### Purpose

- Validate driver fit

- Verify tolerances

- Test recess depth

- Avoid full baffle reprints

### Usage

Set:

export_mode = 4
fitTestDriverIndex = <driver>

Only the puck is rendered.

Print it, test the driver, then proceed with confidence.

## 15. Export Modes

| Mode | Output |
| --- | --- |
| 0 | Full assembly |
| 1 | Box only |
| 2 | Back panel only |
| 3 | Baffle only |
| 4 | Fit test puck |

Always use F6 before exporting STL.

## 16. Recommended Print Strategy

- Print box in multiple sections if needed

- Print baffle face‑down

- Use fit‑test puck before printing baffle

- Dry‑fit everything before final assembly

## 17. Philosophy & Design Intent

G.U.M.B.O. is not a wizard. It is a tool for people who want control.

Everything is explicit. Nothing is hidden. If something looks wrong, the parameters explain why.

## 18. Final Notes

- Keep parameters.scad clean

- Trust the fit‑test puck

- Change one thing at a time

- Save known‑good parameter sets

Once dialed in, G.U.M.B.O. lets you design speakers with confidence instead of guesswork.

End of Manual